Exacq Technologies, headquartered in Indianapolis, Indiana, is a leading developer of open architecture, Video Management System (VMS) solutions for security and surveillance applications. Our exacqVision VMS client-server solutions are scalable from a small single camera solution to large scale corporate or campus systems with thousands of cameras. Real-time and recorded video can be viewed, managed and configured from any location on the network.

For additional information, contact:

Exacq Technologies
	11955 Exit Five Parkway
	Fishers, IN 46037  USA

Phone: +1 317 845-5710

Web: www.exacq.com

E-mail: exacqinfo@tycoint.com

POE+ NETWORK VIDEO RECORDER

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Electronic Surveillance

28 23 00 	Video Surveillance

28 23 19	Digital Video Recorders and Analog Recording Devices

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

POE NETWORK VIDEO RECORDER

- GENERAL

  - SUMMARY

## Section includes a device to power and to acquire, record, store, and display video signals from 4 or 8 IP network video cameras or encoders.

## Related Requirements

      - 28 23 13 - Video Surveillance Control and Management Systems

      - 28 23 23 – Video Surveillance Systems Infrastructure

      - 28 23 29 - Video Surveillance Remote Devices and Sensors

  - REFERENCES

    - Abbreviations

      - PoE – Power over Ethernet

      - HDD – Hard Disk Drive

      - IP – Internet Protocol

      - LDAP – Lightweight Directory Access Protocol

      - Mbps – Megabits per second

      - NVR – Network Video Recorder

      - POS – Point of Sale

      - PSIM – Physical Security Information Management

      - VMS - Video Management System

    - Reference Standards

      - Institute of Electrical and Electronics Engineers (IEEE) 802.3 standards

      - FCC – Code of Federal Regulations, Title 47, Part 15

      - ISO / IEC 14496 – 10 – MPEG-4, Part 10 (H.264)

      - UL

      - CE

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

  - LICENSES

    - The NVR shall have IP camera licenses included, to coincide with the number of provisioned POE+ IP camera/encoder ports.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited three-year warranty and updates for device firmware and client and web software during the warranty period.

      - An extended support option shall be available.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Exacq Technologies

11955 Exit Five Parkway

Fishers, IN 46037 USA

Phone: +1 317 845-5710

Web: www.exacq.com

E-mail: exacqinfo@tycoint.com

    - Model:		M-Series

    - Alternates: 		None

  - DESCRIPTION

    - The PoE Network Video Recorder (“NVR”) shall be an appliance to power and to acquire, record, store, and display video signals from <4> <8> IP network video cameras or encoders.

    - The NVR appliance hardware shall have the following characteristics:

      - Camera/encoder connection

        - <4> <8> IP video cameras or encoders

        - Power

          - PoE (per IEEE 802.3af)

          - 15.4 watts per port

          - Maximum:	<30 watts total for 4 ports> <60 watts total for 8 ports>

      - Storage capacity:	<1 TB> <2 TB>

The product provides 1 HDD, sized at <1TB for 4 cameras> or <2 TB for 8 cameras>. Contact Exacq for configurations and part numbers.

      - Video compression:	H.264, MJPEG

      - Audio

        - Microphone input:	1 RCA connector

        - Outputs:		1 stereo (RCA connectors)

      - Server characteristics:

        - Operating system:	Linux

        - Processor: 		HiSilicon 3535

        - RAM		512 MB

        - Monitor outputs:	HDMI, VGA (mirrored output)

        - USB ports:		3

        - Network Interface:	2x 1000 BASE-T (Gigabit Ethernet)

        - Alarm inputs: 	<4 inputs (TTL)> <8 inputs (TTL)>

        - Alarm outputs:		1 output (relay)

      - Enclosure

        - Material:			Painted steel

        - Dimensions (l x w x h):	13 in. x 8.75 in. x 1.75 in. (33 cm x 22.2 cm x 4.4 cm)

        - Weight: 			9 lbs. (4 kg) maximum

      - Electrical

        - Input voltage:		12V-DC, 7.5A

    - Video Management System (“VMS”)

      - The NVR shall come pre-loaded with VMS server software.

      - The VMS server software shall provide the following features as a minimum:

        - System

          - One server connection per client

          - Browser-based viewing of live and stored video

          - Auto detection of supported cameras

          - Client bandwidth throttling

          - Soft triggers

          - Pre and post alarm recording

          - Continuous motion, time or alarm-based recording, configurable per camera

        - Power over Ethernet (PoE)

          - Power cycle individual POE ports

        - Live video view

          - PTZ control and presets

          - Digital PTZ control and presets

          - Motion and alarm indication

          - Event linking on discrete inputs

        - Search, playback, export, archive

          - Instant replay

          - Event search

            - Thumbnail views

            - Timeline views

          - Multi-camera playback

          - Export options

            - .AVI or.EXE file

      - The NVR shall have the ability to support upgraded VMS software providing additional advanced functionality, including the following:

        - System

          - Server connections – up to 128 via a thick client interface or 4 via web client

          - Ability to specify minimum and maximum retention times on a per camera basis

          - Time-lapse recording

          - Extended storage

          - Archiving

          - Audit trail

          - Custom user groups

          - Intelligent search

          - E-mail notifications on system health

          - Enterprise level camera, server, and user management

          - LDAP and active directory support

        - Live view

          - Event linking on video, serial, and health events

          - Video wall support

          - Event-driven and time-based video switching

          - Camera groups

          - Multi-streaming

          - Event notifications

          - Map support, including hierarchical maps

          - Two-way audio

        - Search, playback, export, archive

          - Multiple camera export

The NVR comes with the exacqVision Start software pre-loaded. Additional functionality is available through upgrade to exacqVision Professional or exacqVision Enterprise VMS software.

    - User Interfaces – The NVR shall support both thick client browser-based and a mobile web client interface.

      - Thick client

        - Client software shall be downloadable at no charge from the NVR Manufacturer’s web site and be fully compatible with all available features of the VMS server software.

        - The client software shall be available for Windows, Apple iOS, and Linux operating systems.

      - Mobile web client

        - A free mobile application shall be available permitting remote view of live and recorded video through the NVR.

        - The mobile application shall support PTZ control and the monitoring and activation of alarms and triggers for the mobile device.

        - The mobile application shall be available for devices running Apple iOS, Google Android, and Amazon Kindle Fire software.

        - The mobile application shall allow simultaneous interaction with multiple NVR devices from the same Manufacturer.

        - The web service supporting the mobile application shall size the video stream to accommodate both low bandwidth and high bandwidth networks.

  - PERFORMANCE

    - Compatibility

      - Video

        - The NVR shall have the ability to connect to Tyco Illustra, Arecont and Axis cameras prior to any initial camera settings and initiate video recording without modification of the client or camera settings.

        - The NVR shall have the ability to connect to all cameras listed at https://exacq.com/integration

      - Access control - The NVR shall be compatible with the following access control manufacturers:

      - POS and retail analytics - The NVR shall be compatible with the following POS and retail analytics manufacturers:

      - PSIM - The NVR shall be compatible with the following PSIM manufacturers:

Specifier should complete the above sections to include manufacturers of those existing or new devices or software that will interface with the NVR.

A list of supported integrations from Exacq is available at https://exacq.com/integration.

    - Recording

      - Video storage rate:		40 Mbps

    - Display

      - Local client display rate:		120 frames per second

      - Local client display resolution	Max 1936x1936 camera resolution

  - ENVIRONMENTAL

    - Operating temperature:		40 – 95 degrees Fahrenheit (4.5 -  35 degrees C)

END OF SECTION

- EXECUTION

  - INSTALLATION

    - Contractor shall comply with all Manufacturer installation guidelines.

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - STORAGE

    - Hardware shall be stored in an environment where temperature and humidity are in the range specified by the hardware manufacturer.

END OF SECTION